Change Management Log

Firewall Hardening — All Three Network Zones

1. Change Details

| Change ID | CHG-2026-010 |
| --- | --- |
| Change Title | Firewall Hardening — All Three Zones (Servers / Clients / Attack-DMZ) |
| Initiated By | Lab Administrator |
| Date Submitted | February 28, 2026 |
| Implementation Date | February 28, 2026 |
| Environment | Home Lab — lab.local (VMware Workstation Pro) |
| Firewall Platform | pfSense 2.7.2 — SRV-FW01 (10.10.10.1 / 10.10.20.1 / 10.10.30.1) |
| Change Category | Security — Network Access Control |
| Priority | High |
| Change Type | Standard (Planned) |
| Status | Implemented — Verified |

2. Change Summary

This change implements explicit firewall rules across all three network zones in the lab environment (Servers — VMnet10, Clients — VMnet11, Attack/DMZ — VMnet12) using pfSense 2.7.2. Prior to this change, inter-zone traffic was controlled only by pfSense default-deny on WAN with no explicit rules restricting lateral movement between internal zones.

The objective of this change is to enforce network segmentation consistent with enterprise security best practices, reduce the attack surface for lateral movement techniques (T1021), and ensure that the lab environment accurately reflects a hardened production network for SOC simulation and detection engineering purposes.

Ten firewall rules were created covering all inter-zone traffic paths. Each rule was risk-justified, applied with least-privilege intent, and verified through live testing with Wazuh SIEM monitoring active.

3. Scope of Change

Systems Affected:

- SRV-FW01 — pfSense 2.7.2 (primary change target — all rule additions)

- ATT-KALI01 — 10.10.30.130 (Attack/DMZ zone — outbound traffic now restricted)

- DMZ-VULN01 — 10.10.30.131 (Attack/DMZ zone — only reachable target from Attack zone)

- CLI-WIN11-01 — 10.10.20.134 (Clients zone — restricted to AD auth ports toward Servers)

- SRV-DC01, SRV-DB01, SRV-SIEM01 — 10.10.10.x (Servers zone — inbound from Attack zone blocked)

Out of Scope:

- Physical network infrastructure (TP-Link TL-SG108E — planned Phase 14)

- WAN-side rules beyond default deny inbound

- Application-layer (Layer 7) filtering

4. Firewall Rules Implemented

| Interface | Source | Destination | Port / Proto | Action | Justification / MITRE Ref |
| --- | --- | --- | --- | --- | --- |
| OPT2 (Attack) | 10.10.30.0/24 | 10.10.30.131 only | Any | PASS | Allow Attack zone → DMZ-VULN01 only — contains lateral movement to servers (T1021) |
| OPT2 (Attack) | 10.10.30.0/24 | 10.10.10.0/24 | Any | BLOCK | Block Attack → Servers zone — prevents T1021 lateral movement and T1046 internal recon |
| OPT2 (Attack) | 10.10.30.0/24 | 10.10.20.0/24 | Any | BLOCK | Block Attack → Clients zone — prevents pivoting to workstations post-exploitation |
| OPT1 (Clients) | 10.10.20.0/24 | 10.10.10.0/24 | 53/88/389/445/3389 | PASS | Allow Clients → Servers on AD auth ports only — principle of least privilege; limits T1078 abuse |
| OPT1 (Clients) | 10.10.20.0/24 | Any | Any | PASS | Allow Clients → Internet — required for updates, browsing |
| OPT1 (Clients) | 10.10.20.0/24 | 10.10.30.0/24 | Any | BLOCK | Block Clients → Attack zone — prevents reverse pivoting from client compromise |
| LAN (Servers) | 10.10.10.0/24 | 10.10.20.0/24 | Any | PASS | Allow Servers → Clients — needed for AD services, Group Policy, DNS resolution |
| LAN (Servers) | 10.10.10.0/24 | Any | Any | PASS | Allow Servers → Internet — required for Windows Update, NTP, DNS |
| LAN (Servers) | 10.10.10.0/24 | 10.10.30.0/24 | Any | BLOCK | Block Servers → Attack zone — prevents C2 callbacks and exfiltration to attack host (T1041) |
| WAN | Any | Any | Any | BLOCK | Default deny all inbound — no services exposed externally; baseline perimeter control |

5. Risk Assessment

| Risk / Concern | Severity | Mitigating Controls | Decision |
| --- | --- | --- | --- |
| Attack zone can reach internet via WAN | Medium | Monitored via Wazuh; isolated on VMnet12; no production data | Accepted — lab requirement for tool updates |
| pfSense GUI accessible from LAN (VMnet10) only | Low | GUI not exposed to Clients or Attack zones | Accepted — access restricted by firewall zone design |
| Clients zone allowed outbound to internet | Low | Required for realistic domain client behavior; Wazuh agent monitoring active | Accepted — necessary for lab realism |
| Lateral movement blocked but not fully simulated post-rule | Low | Rules verified via Wazuh alerts during live attack scenarios | Accepted — Phase 8 SOC scenarios confirmed detection |

6. Rollback Plan

In the event that the implemented firewall rules cause unintended connectivity loss or break required lab functionality, the following rollback procedure applies:

- Access pfSense Web GUI at https://10.10.10.1 using credentials: admin / pfSense2025Lab!

- Navigate to Firewall → Rules and identify rules associated with this change (CHG-2026-010).

- Disable or delete the specific rules causing the issue.

- Verify connectivity is restored by testing from affected VMs.

- Document the rollback in this log with timestamp and reason.

- A full configuration backup was exported from pfSense prior to this change and stored locally.

7. Post-Implementation Verification

All rules were verified through live testing immediately following implementation. Wazuh SIEM (SRV-SIEM01) was active with all 4 agents reporting throughout the verification process.

| Test | Method | Tool / Evidence | Result |
| --- | --- | --- | --- |
| OPT2 → Servers BLOCK | Ran nmap from ATT-KALI01 (10.10.30.130) targeting 10.10.10.128; all ports filtered; no response received | ATT-KALI01 terminal + Wazuh | PASS |
| OPT2 → Clients BLOCK | Attempted ping and port scan from ATT-KALI01 to 10.10.20.134; all traffic dropped | ATT-KALI01 terminal | PASS |
| OPT2 → DMZ-VULN01 PASS | Confirmed nc 10.10.30.131 1524 shell access from ATT-KALI01; Metasploit module exploits functional | ATT-KALI01 terminal | PASS |
| OPT1 Clients → Servers (AD ports) | Domain join, GPO application, RDP to DC all functional from CLI-WIN11-01 | CLI-WIN11-01 Event Viewer + DC | PASS |
| OPT1 Clients → Attack BLOCK | Ping and traceroute from CLI-WIN11-01 to 10.10.30.130 — all dropped | CLI-WIN11-01 terminal | PASS |
| LAN Servers → Attack BLOCK | Confirmed outbound from SRV-DC01 to 10.10.30.0/24 is blocked — T1041 C2 path closed | SRV-DC01 + Wazuh | PASS |
| WAN inbound BLOCK | No services reachable from VMnet8 WAN side; pfSense default deny confirmed active | pfSense firewall logs | PASS |
| Wazuh alert on blocked traffic | Wazuh Rule 1002 fired on repeated blocked attempts from ATT-KALI01 — confirmed SIEM visibility | Wazuh Dashboard | PASS |

8. Approval & Sign-Off

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Change Implementer | Lab Administrator | February 28, 2026 | Lab Environment — Self-Approved |
| Change Reviewer | Lab Administrator | February 28, 2026 | Lab Environment — Self-Approved |
| Lab Owner | Lab Administrator | February 28, 2026 | Lab Environment — Self-Approved |

9. Lessons Learned & Notes

- pfSense evaluates rules top-to-bottom per interface — rule ordering matters. Specific PASS rules for DMZ-VULN01 were placed above the broader BLOCK rules to ensure they matched first.

- The Clients zone (OPT1) requires port-specific PASS rules for AD authentication (DNS/88/389/445/3389) before any broad BLOCK rules. Without this, domain logins and Group Policy fail.

- Wazuh alert visibility on blocked traffic confirmed that SIEM coverage extends to network-layer events, not just host-based telemetry — critical validation for SOC simulation.

- This change directly supports MITRE ATT&CK detection coverage for T1021 (Remote Services), T1046 (Network Service Scanning), T1041 (Exfiltration Over C2), and T1078 (Valid Accounts).

Change Management Log — CHG-2026-010 | lab.local Home Lab | February 28, 2026